# Appendix 1: Data standards

# 1. Spatial Data

# Topology

Data will be checked for topological errors such as dangling lines, sliver polygons and undershoots (bleeding polygons).  Where possible, these errors will be corrected through a process of topology validation.

# Projection

Data will be projected into the following projection:

Projection: Transverse Mercator

Spheroid: Clarke1880

Datum: Cape

Central Meridian: 31 00 0.00

Reference Latitude: 0 0 0.00

Scale Factor: 1

False Easting: 0.00

False Northing: 0.00

# Formats

All spatial data will be provided in ArcView shapefile format.

# 2. Attribute Data

If the dataset does not already have a unique identifier field, a new one will have to be assigned and populated with unique numbers for each record. This field must be renamed using the feature (dataset) name plus “ID”, e.g. the Roads dataset is named glRoad, and the ID field is named RoadID.

- Field Naming

- No punctuation or spaces in field names.

- Words are distinguished by capitals, e.g. SectionName.

- Abbreviations are avoided (if possible), except for ID (Identifier), e.g. SectionID (Section identifier).

- Data Definition

- The Identifier field is stored as Long Integer when numeric, or a string of fixed length when necessary.

- Areas (Plan or Surface) are always stored as a double (Double Precision Floating Point).

- ID numbers) are stored as Long Integers.

- Comments and Descriptive text fields are stored as Text of 255 characters.

- Dates are always stored in Short Date format.

- Costs are stored as Currency – General Number format.

- Other data should be stored in the most appropriate format, to maximise efficiency, eg. Road Names should not be stored as 255 character Text, as it is unlikely that the name will ever require more than 50 characters. A field to indicate whether or not a plant is rare, should be stored as Yes/No.

# 3. Metadata

The following metadata fields are suggested for the data, these are based on the Federal Geographic Data Committee (FGDC) metadata standards.

| Field Name | Explanation |
| --- | --- |
| Citation Title | Full name by which data set is known. |
| Abstract | A brief narrative summary of the data set. |
| Purpose | A summary of the intentions with which the data set was developed. |
| Supplemental Information | Other descriptive information about the data set. |
| Access Constraints | Restrictions & legal prerequisites for accessing the data set. |
| Use Constraints | Restrictions & legal prerequisites for using the data set after access is granted. |
| Data Set Credit | Recognition of those who contributed to data set. |
| Originator | The name of an organization or individual that developed the data set. |
| Contact Organisation | The organisation to be contacted to access the data. |
| Contact Person | No necessarily the person who developed the data. |
| Position | Position of contact person |
| Telephone | Telephone number of contact person |
| Fax | Fax number of contact person |
| E-mail | E-mail address of contact person |
| Hours of service |  |
| Mailing address | Mailing address of organisation |
| Publication Date | The date when the data set is published or otherwise made available for release. |
| Calendar Date | Date of original capture. |
| Currentness Reference | The basis on which the time period of content information is determined. Choose from: ground condition or publication date. |
| Edition | Version of the title. |
| Series Name | Name of series published of which data set is a part. |
| Progress | The state of the data set. Select from: complete, in work, planned. |
| Update Frequency | The frequency with which changes and additions are made to the data set after the initial data set is completed. Select from: annually, as needed, continually, daily, irregular, monthly, none planned, quarterly, unknown, weekly |
| Keywords | Common-use word or phrase used to describe the subject of the data set. |
| Logical Consistency Report | An explanation of the fidelity of the relationships in the data set and the test used. |
| Completeness Report | Information about omissions, selection criteria, generalisations, definitions used, and other rules used to derive the data set. |
| Attribute Accuracy Report | An explanation of the accuracy of the identification of the entities and assignments of values in the data set and a description of the tests used. |
| Attribute Accuracy Value | An estimate of the accuracy of the identification of the entities and assignments of attribute values in the data set. |
| Attribute Accuracy Explanation | The identification of the tests that yielded the Attribute Accuracy Value. |
| Attribute Descriptions | Descriptions (including code descriptors) for each of the attribute fields used. |
| Horizontal Positional Accuracy Report | An explanation of the accuracy of the horizontal co-ordinate measurements and a description of the tests used. E.g., digitised on screen off 1:10,000 map using ArcView 3.2 - (accuracy statement based on source map/data and method of capture). |
| Horizontal Positional Accuracy Value | An estimate of the accuracy of the vertical co-ordinate measurements in the data set expressed in (ground) meters. E.g., 10 (meters). |
| Horizontal Positional Accuracy Explanation | The identification of the test that yielded the Vertical Positional Accuracy Value. E.g., based on source map scale. |
| Vertical Positional Accuracy Report | An explanation of the accuracy of the vertical co-ordinate measurements and a description of the tests used. (Only applicable for data such as DEMS). E.g., Based on method of interpolation of sample height values. |
| Vertical Positional Accuracy Value | An estimate of the accuracy of the vertical co-ordinate measurements in the datas et expressed in (ground) meters. (Only applicable for data such as DEMS). E.g., 50 (metres) |
| Vertical Positional Accuracy Explanation | The identification of the test that yielded the Vertical Positional Accuracy Value. (Only applicable for data such as DEMS). E.g., IDW/Kriging. |
| Source Scale Denominator | Scale of capture. |
| Process Description | Describe all processes used and organisations/people responsible for getting data to its current form. |
| Process Software | The name and version of software used to process the data from which this data is derived. For example: ArcInfo version 8.0. |

# 4. File Naming Conventions

Files and tables will be named according to the following 8 character naming convention.

The first character defines the object type.

g = geospatial object (Shape file and associated shx and dbf files)

r = related attributes for geospatial object (Table within Access, 1-to-1 relationship with geospatial object)

l = lookup table (Table of code definitions within Access)

t = table (Table linked to a Related Attribute table rather than a Geospatial Object, in Access)

k = link table (A table used to define a many-to-many relationship)

The second character defines the scale at which the data were captured (This is used only with geospatial objects and tables of related attributes)

l = local (1:10 000)

r = regional ( - 1:50 000)

p = provincial ( - 1:250 000)

n = national (- 1:1 000 000)

e.g. ‘grGeol.shp’ is an ArcView shapefile of geology at 1:50 000. ‘rrGeol’ is a table of attributes in the Access database.

The following 6 characters will describe the dataset in short.